SBUHB Loss of Utilities (gas, electricity, water) Multi Utility Failure Framework

| DOCUMENT AUTHOR: | ## SBUHB Loss of Utilities (gas, electricity, water) Multi Utility Failure Preparedness & Response Group |
| --- | --- |
| APPROVED BY: | EPRR Oversight Group |
| APPROVAL DATE: | 7th May 2026 |
| REVIEW DATE: | 7th May 2029 |
| DOCUMENT No: | v1 FINAL |

| Policy Version Number | Date | Author | Description of Change |
| --- | --- | --- | --- |
| 0.1 | February 2026 | Head of EPRR | Initial Draft |
| 0.2 | 20th March 2026 | EPRR Manager | Change of framework name and alignment with updated ToR |
| 0.3 | 15th April 2026 | HB Loss of Utilities (gas, electricity, water) multi utility failure Preparedness & Response Group | Framework created as working DRAFT |
| 1 | 20th April 2026 | HB Loss of Utilities (gas, electricity, water) multi utility failure Preparedness & Response Group | Final version subject to Board endorsement and approval |

CONTENTS

- Purpose

- Scope

- Risk Content

- Maintenance, Governance & Accountability

- Command, Control & Co-ordination (C3) & JESIP Interoperability Principles

- Roles and Responsibilities

- Detection and Alerting

- Immediate Response

- Sustained Outage Management

- Vulnerable Population, Critical Services & Clinical Prioritisation

- Contractor Control and Safety

- Recovery and Stand-Down

- Learning and Assurance

- Training and Exercising

- Document References

- Glossary

- Appendices:

A 	Power outage decision log

B 	Generator status checklist

C   Contractor Permit to Work template

D   Crisis Communications Plan

E   Fuel monitoring log

F   Preparing for and Responding to Energy Emergencies

G	National Fuel Plan

H	Public Health Advice – Water & Sanitation during a national power outage

I	UK Government Guidance: Written Science Annex - National Power Outage

J	Health Technical Memorandum 06-01: Electrical services supply and distribution

K	National Emergency Plan – Downstream Gas & Electricity

L	South Wales Police MACG

M  JESIP Interoperability Principles & Joint Decision-Making Model

# HEALTH BOARD EMERGENCY RESPONSE, COMMAND, CONTROL & CO-ORDINATION (C3)

The following will be a standard process as articulated within the Health Board (HB) Major Incident (MI) Procedures or BC/Critical incident Procedures for any Emergency Response:

- C3 arrangements

- JESIP Interoperability Principles

- Information management

- Communication Strategy

- Service Business Continuity Procedures to be invoked alongside any Emergency Response

All Emergency Response Procedures are flexible and adaptable; this allows for a tailored response to all emergencies and any additional threats and risks that may arise at the time.

## PURPOSE

This framework sets out how Swansea Bay University Health Board (SBUHB) will prepare for, respond to, and recover from a partial or total loss of Utilities (gas, electricity, water, disruption or contamination) affecting any SBUHB site. This could be as a result of a local, regional or national outage.

Its purpose is to:

- Protect patient safety and clinical services

- Ensure effective command, control and coordination across Estates, Emergency Preparedness, Resilience & Response (EPRR) and clinical services

- Provide clear escalation triggers and responsibilities

- Strengthen organisational resilience to partial or total loss of utilities related incidents

- Impacts on healthcare facilities, community care, vulnerable populations, and critical infrastructure

- SCOPE

This framework applies to:

- All SBUHB-owned or managed sites including:

  - Morriston Hospital

  - Singleton Hospital

  - Neath Port Talbot (NPT) Hospital

  - Mental Health & Learning Disabilities Facilities

  - Primary & Community hospitals and health centres

- All forms of utilities disruption/contamination, including:

  - Loss of power

  - Loss of water supply

  - Loss of Gas

  - National Grid / Distribution Network Operator (DNO) failures

  - Internal distribution or substation failures

  - Generator failures or unintended generator running

  - Fuel supply failures

  - Planned electrical works escalating into incidents

  - Multi utility failure

- RISK CONTEXT

Loss of utilities presents a critical risk to patient safety and service delivery across Swansea Bay University Health Board (SBUHB), impacting acute hospitals, community and primary care services, mental health settings and care delivered in patients’ homes. Modern clinical services are highly dependent on reliable power, water, medical gases, digital systems and controlled environments; disruption to any of these can rapidly compromise emergency care, critical care, diagnostics, dialysis, theatres, medicines management and infection prevention.

Utility failures may occur without warning, cascade across sites and systems, and coincide with wider infrastructure or digital disruption, significantly increasing the risk of service suspension, patient displacement and escalation to a major or critical incident. Effective clinical prioritisation, timely escalation and coordinated command and control are therefore essential to mitigate harm and maintain continuity of care.

Electrical supply underpins:

- Life sustaining clinical services and equipment

- Theatres, laboratories, mortuary fridges, decontamination and sterilisation facilities

- Telephony systems for internal and external calling, and contact centres

- Communications,  diagnostic and imaging systems

- Medical gases, lifts, ventilation and environmental controls

- Pharmaceuticals

- Digital Infrastructure (HB data centres & external providers)

- Digital Platforms, including desktops and laptops

- Renal dialysis functions would be unable to function

- HB security functions (SALTO)

Loss of power may:

- Occur without warning

- Cascade across sites or systems

- Coincide with wider infrastructure disruption (fuel, telecoms)

Loss of Water Supply risks

- Inability to maintain hygiene

- Operating theatres cannot safely function due to inability to scrub or sterilise equipment

- Laboratories analysers will be unable to function

- Impact on heating and cooling systems, boilers and sterilisation units dependent on water

- Renal dialysis functions would be unable to function

Loss of Heating, ventilation and Air Conditioning (HVAC) risks

- Patient safety, infection control and imaging equipment

- Temperature and humidity deviations which can compromise surgical environment, sterile supplies and drug stability

- Operating theatres, Cardiac Intensive Therapies Unit (CITU), Cardiac High Dependency Unit (CHDU), General Intensive Therapies Unit  (ITU), Neonatal Units, Maternity, Wards, Minor Injuries Unit (MIU), Radiology, Enhanced Care Unit (ECU)could not safely function

- Laboratories analysers will be unable to function

Loss of Medical Gases (Oxygen, Medical Air, Vacuum) risks

- Immediate threat to life for ventilated or oxygen dependent patients

- Inability to conduct safe surgery or anaesthesia which rely on medical air and vacuum

- Clinical deterioration across, Emergency Department (ED), Acute Medical Unit (AMU), ITU, Theatres, Neonatal Units, Maternity, Wards, Minor Injuries Unit (MIU), Radiology, Enhanced Care Unit (ECU)

Loss of Fuel (for Generators, Boilers, Emergency Transport) risks

- Failure of standby generator power

- Inability to maintain heating/hot water from dual fuel oil fired boilers

- Impacts on mortuary refrigeration, pharmacy storage and temperature-controlled laboratories

Loss of  Digital & Telecommunications systems risks Loss of access to clinical systems, including digital patient records, diagnostic systems, e-prescribing, theatre systems, pathology and radiology.

- Clinical decision-making compromised due to lack of real-time data.

- Communication breakdown (telephony and cisco phones, emergency alerting, external calling), slowing emergency response and coordination

- Loss of inter-hospital connectivity resulting in loss of access to Microsoft 365 services, national systems and internet hosted services.

Multiple Utilities Lost Together risks

- Cascading failures

- Loss of water           loss of boilers          loss of heating

- Loss of power          failure of IT, pumps, ventilation, medical gas plant

- Concurrent or sequential failures especially following extreme weather or infrastructure damage.

These scenarios increase the likelihood of major service disruption and evacuation.

Reputational, Operational & Safety Risks beyond clinical impacts

- Need to cancel surgery and appointments, leading to backlog and patient harm

- Reduced emergency department throughput causing overcrowding risk

- Regulatory and legal consequences, breaching statutory requirements

- Reputational damage and loss of public trust

This framework builds on:

- HTM / WHTM 06-01- Electrical Services Supply and Distribution (engineering guidance on the design, installation, operation and maintenance of electrical) supply and distribution systems in healthcare premises) (Appendix J)

- EPRR Core Standards

- Learning identified through recent SBUHB incidents and near misses

Hazard Identification

- Severe weather (storm damage, flooding, cold spells)

- Infrastructure failure

- Cyber attack

- Fuel shortages

- Supply chain

- Industrial action

- A major incident could be the result of, or a concurrency of the above points

National frameworks emphasise ongoing risk monitoring and governance structures to track threats to gas and electricity systems (Appendix K).

Consequence Assessment

Consider:

- Risks to patient harm or safety

- Loss of heating and critical equipment

- Reduced water pressure affecting clinical services

- Potential contamination

- Failure of telecommunication systems

- Increased demand on emergency services

- MAINTENANCE, GOVERNANCE & ACCOUNTABILITY

The framework is subject to a 3 yearly review to ensure it remains fit for purpose and that any changes in guidance or legislation are reflected in the procedure. This process also enables any lessons identified from live incidences or training and exercising to be incorporated.

Following a review, the procedure is submitted to the Emergency Preparedness, Resilience and Response, (EPRR) Oversight Group, Executive Team Board and Health Board for endorsement and approval. Once approved, the procedure is circulated appropriately within the Health Board.

### Executive Accountability:

- Executive Lead: Director of Estates

- Professional Leads:

  - Head of Estates & Facilities

  - Head of Emergency Preparedness, Resilience and Response

### Assurance:

- Oversight through:

  - Infrastructure / Estates Governance structures

  - EPRR Oversight Group

- Escalation, endorsement and approval to:

  - Executive Team Board

  - Health Board

- COMMAND, CONTROL & CO-ORDINATION (C3) & JESIP INTEROPERABILTY PRINCIPLES

No matter how severe the impact of the incident the same management arrangements will be established within the Health Board as noted above.

This will include Operational (bronze) and Tactical (Silver) meetings, and if necessary, Gold command and control. The operational response structures will be as noted within both the associated emergency response procedures and action cards. (Please note, there may be a need for some areas to reference action card sets from each of these procedures).

The JESIP Interoperability Principles should be used during all phases of an incident, whether spontaneous or pre-planned and regardless of scale (Appendix M).

They support the development of a multi-agency response and provide structure during the response to all incidents. The principles can also be applied during the recovery phase.

To ensure commanders bring together all of the available information, reconcile potentially differing priorities and then make effective decisions, the Joint Decision Model (JDM) should be used (Appendix M).

The JDM centres around three primary considerations:

Incident Levels

| Level |  | Description |
| --- | --- | --- |
| Local Estates Incident |  | Short-duration power interruption managed locally via Business Continuity Management (BCM) arrangements |
| Site Incident |  | Loss of essential supply to clinical areas, service BCMs, site overarching/tactical Business Continuity Procedures (BCPs), Silver command & control – establishment of hospital/incident co-ordination centres (H/ICC) |
| HB Critical Incident |  | Sustained outage or multiple sites affected, HB wide Silvers, overarching Silver & Gold command. |
| Major Incident |  | Widespread or prolonged loss of power requiring external support via national & regional civil contingencies & command structures |

Clear escalation triggers include:

  - Loss of power

  - Loss of water supply

  - Loss of Gas

  - National Grid / Distribution Network Operator (DNO) failures

  - Internal distribution or substation failures

  - Generator failures or unintended generator running

  - Fuel supply failures

  - Planned electrical works escalating into incidents

All Incident Response Levels in the table below will be used by SBUHB to ensure consistent notification, escalation and co-ordination of emergency incidents.

| 1 | A business continuity that can be locally managed, (at a specific service level), without invocation of a declared business continuity incident. This includes the actions noted in the NHS Risk Assessment Escalation Action Plan. Each service, inclusive of Estates will be required to adopt policies and procedures consistent with the extreme weather that is anticipated and experienced. |
| --- | --- |
| 2 | A business continuity/emergency incident that requires invocation of specific service(s) and Service Delivery Group Business Continuity Procedures (BCPs) to ensure tactical incident command, control and co-ordination, (SDG Silver Command). This includes 3&4; Amber/Red levels in the NHS Risk Assessment Escalation Action Plan. The resource levels will depend on the scale of the incident and will be determined as necessary. |
| 3 | A business continuity/critical incident that requires invocation of more than one Service Delivery Group/specialties business continuity, requiring invocation of the SBUHB Overarching Business Continuity Procedure to provide strategic co-ordination, (HB Gold Command). This includes black level, (risk score 25) in the NHS Risk Assessment Escalation Action Plan. |

There will be three levels of Business Continuity/Critical Incident activation, and this will provide the most efficient structure for incident response and recovery. It will enable incidents to be managed at the lowest level for smaller scale incidents, whilst allowing for escalation through the levels of management for a larger more severe incident and these are noted above. All services will need to invoke their BCPs at level 1.

The Estates BCP includes the following:

- Electricity Outage Plan

- Generator start-up procedures

- Fuel management & resupply

- Prioritisation of clinical services

- Black Start / Electricity Service Restoration principles (as referenced in the London Power Supply Framework).

Water Outage Plan

- Activation of alternative water provision (bowsers, bottled water)

- Clinical water use prioritisation matrix

- Toilet & sanitation contingency

- Coordination with the Drinking Water Inspectorate (DWI) and water utility resilience teams

Gas Supply Emergency Plan - Based on Health & Safety Executive (HSE) and International Genetically Engineered Machine

(IGEM) guidance:

- Decision trees for safe shutdown of gas-fired equipment

- Protection of vulnerable individuals during cold weather

- Procedures for large gas consumers, including shutdown and safe restart.

Multi Utility Failure Plan

Address interdependencies:

- Telecoms resilience (VoIP / mobile network outage)

- Information Technology (IT)/data centre continuity

- Security systems failure

- Transport impacts for staff and patients

National Fuel Plan Considerations

In the of a major fuel supply disruption affecting the United Kingdom (UK) the National Fuel Plan (Appendix G) details a wide range of measures, both statutory and non-statutory, that can be implemented by government to support in responding to any disruption. The principal aim of the measures is to maintain fuel supplies as close to normal levels as possible.

Where fuel resilience risks are identified beyond local mitigation arrangements, escalation through Health Board command structures and, where appropriate, via SWLRF multi-agency coordination arrangements will be undertaken to support prioritisation, mutual aid and national situational awareness.

Health Board Command Structure

- Gold: Executive / Strategic

- Silver: Tactical coordination

- Bronze: Operational delivery

Estates retains technical authority

A nationally recognised three-tiered command and control structure known as Strategic (Gold), Tactical (Silver) and Operational (Bronze) has been adopted by the emergency services and most responding agencies. The Health Board command and control arrangements are based upon this system.

Health Board Gold command is fully detailed in section 2 of the SBUHB Overarching Business Continuity/Critical Incident Procedure. However, the level of command required will be determined by the nature of the incident and activated in accordance with the levels as noted above.

It is imperative to note that if a Level 3 Business Continuity Incident is declared that the whole Health Board must be fully aware that it responding to an emergency and non-critical Health Board activity may need to cease and on occasion’s additional staff from non-clinical areas may be required to assist.  This will be decided at a Strategic, (Executive) level.

Health Board Gold, (Strategic) Command Structure

Dependent upon the nature of the incident a Gold Command Team may be convened. The decision to convene a Health Board Gold Command will be made by the Executive on Call at the time of the incident and following a review of the incident details and will be in accordance with a Level 3 incident as noted above.

This level determines the co-ordinated strategy and policy for overall management of the incident. This level of management also formulates media handling and public communications strategies as required and necessary. The Strategic Lead will then delegate actions to the respective Service Delivery Group Tactical (Silver) control level for them to implement a tactical plan to achieve the Strategic aim and objectives.

The Gold Team may comprise of the following, this will depend on the incident;

•	Gold Commander

•	Medical Director

•	Chief Operating Officer

•	Nurse Director

•	Director of Finance

•	Director of Workforce & Organisational Development (WOD)

•	Director of Planning & Partnerships

•	Head of Communications

•	Loggist

There must be consideration for Strategic resilience for protracted incidences.

Tactical (Silver Command)

If a Business Continuity Incident is declared within a Service Delivery Group (SDG), Tactical, (Silver Control) will be required.  If only one SDG is affected, the incident will be managed at a Tactical level.

Silver Control within the Health Board will be convened within the on-site Co-ordination Centres.  They will be responsible for developing and implementing a tactical plan and where the incident affects more than one SDG, to achieve the strategic direction set by Gold command. Health Board Silver will oversee the operational response and will be the link between strategic and operational.

The Silver Commander should have more information available about the incident than anyone else.

The core Silver Command Team will include the Service Delivery Groups (SDG), Nurse and Medical Directors or Deputies and may comprise of the following, this will depend on the incident;

•	Silver Commander

•	Associate Service and Medical Directors

•	Clinical Directors/Deputies

•	Heads of Nursing

•	Assistant Service Directors/Managers

•	Support Services/Estates

•	Support Functions, e.g. Finance, Workforce & Organisational Development (WOD), Communications

•	Site Management

•	Loggist

•	Administrative Support

Operational (Bronze Controls)

These are the locations where resources are deployed to carry out the tasks required in responding to the incident. The operational teams will manage the physical response to achieve the tactical plan. There may be a number of Health Board operational teams included as part of the response and will be organised into groups dependent on the emergency. The operational response to a Business Continuity incident or significant emergency is articulated in the respective Service Delivery Group Business Continuity Procedures and corporate emergency response Procedures.

Multi-Agency Command, Control & Coordination

Where a utilities disruption is assessed to have actual or potential wider cross-sector, geographic or population impacts, Swansea Bay University Health Board (SBUHB) will operate as a Category 1 responder within established South Wales Local Resilience Forum (SWLRF) structures.

Activation of multi-agency arrangements may occur where:

- There is a regional or national infrastructure failure (e.g. electricity, gas, fuel, water or telecoms),

- The impact exceeds Health Board capability to manage in isolation, or

- Coordinated prioritisation across agencies is required to protect life-critical services and vulnerable populations.

In such circumstances, SBUHB may engage with or contribute to one or more of the following multi-agency command structures, convened through the SWLRF via the South Wales Police (SWP) Multi-Agency Co-ordination Group (MACG) Procedure (Appendix L):

- Strategic Coordinating Group (SCG) – providing strategic direction and policy coordination across agencies;

- Tactical Coordinating Group (TCG) – managing cross-agency tactical response and resource prioritisation;

- Recovery Coordinating Group (RCG) – leading multi-agency recovery and restoration activity where required.

- Situational Awareness Multi-Agency Meeting (SAMM) - The role of the SAMM is to gain situational awareness from our multi-agency partners for a rising tide event. The Chair of the initial SAMM will be from the organisation who has called for the multiagency meeting.

SBUHB Gold (Strategic) Command will retain responsibility for Health Board response objectives and internal prioritisation, while ensuring alignment with the multi-agency strategy, JESIP principles and joint decision-making arrangements.

Health Board representation to multi-agency groups will be via the Health Board Gold or Silver Commander, supported by the EPRR function, ensuring consistent situational awareness, information sharing and coordinated decision-making across systems.

Engagement with SWLRF arrangements will be flexed proportionately, recognising that not all utilities incidents will require formal multi-agency escalation.

- ROLES & RESPONSIBILITIES

### This section sets out the roles and responsibilities of key Corporate, Cross Cutting Services, SDGs and Services in preparing for, responding to, and recovering from a loss of utilities or multi‑utility failure incident.

### Clear accountability is essential to ensure timely decision‑making, effective command and control, and the safe continuity of patient care. The responsibilities outlined below apply across all phases of an incident, including preparedness, response, sustained outage management, and recovery, and must be delivered in accordance with established command structures and business continuity arrangements.

### Estates

- Technical management and infrastructure safety

- Technical diagnosis and restoration of utility services

- Generator and fuel management

- Contractor control and Permit to Work

### Chief Operating Officer (COO)

- Executive leadership of operational response

- Operational oversight and Strategic decision making

- liaison with partners and Welsh Government

### EPRR

To meet the 6 statutory duties under the Civil Contingencies Act (CCA) 2004, the EPRR team role includes the following responsibilities for planning, preparedness, response & recovery to any emergency:

- Planning & Preparedness

- Co-ordination & Communication of EPRR within the Health Board (HB)

- Response & Activation Measures

- Resilience & Recovery Processes

- Collaboration with External Agencies

- Monitoring & Evaluation

Morriston SDG

Will support with the planning, preparedness, response & recovery to any emergency by:

- Contributing to Planning & Preparedness

- Co-ordination & Communication of EPRR within your respective services

- Resilience, Escalation,  Response & Recovery Processes, (particularly in terms of business continuity management within your service)

- Collaboration with External Agencies for your respective service workstreams

- Monitoring & Evaluation through HB & service specific debriefs

NPTSSG

Will support with the planning, preparedness, response & recovery to any emergency by:

- Contributing to Planning & Preparedness

- Co-ordination & Communication of EPRR within your respective services

- Resilience, Escalation,  Response & Recovery Processes, (particularly in terms of business continuity management within your service)

- Collaboration with External Agencies for your respective service workstreams

- Monitoring & Evaluation through HB & service specific debriefs

### In addition:

Associate Director / Divisional Manager – Hospital Operations (BCP Lead)

- Coordinates activation of Loss of Utilities Plans and supports Estates/Kier with operational constraints and escalation.

- Supports the setup and operation of the Silver Command Centre during outages.

- Provides operational oversight of patient flow, capacity, safe staffing, and continuity of unaffected areas.

Service Group Director / Deputy (Silver Commander)

- Provides overall command and control for the Service Group during a utilities-related Business Continuity Incident.

- Chairs Silver Command, ensuring tactical coordination across Estates/Kier, Digital, Clinical Teams, and site leadership.

- Ensures all key staff and technical specialists (Estates AP/CP, utilities engineers (including Kier), Digital) are alerted and present.

- Reports upwards to the Executive Team / Gold Command where severe or multi-site disruption occurs.

- Maintains oversight of service prioritisation, risk management, and safe recovery once utilities are restored.

Directorate Managers (Bronze Leads)

- Identify clinical services that are vulnerable to utility failures (e.g., oxygen-dependent units, theatres, diagnostics).

- Lead the local Bronze response, ensuring communications and immediate controls (e.g., equipment shutdown, manual workarounds, relocation) are initiated.

Service Group Medical Director

- Provides senior clinical oversight on risks related to loss of utilities (impact on theatres, critical care, diagnostics, oxygen supply etc.).

- Advises Silver Command on safety, patient prioritisation, clinical suspension, and decanting decisions.

- Ensures serious incidents arising from the outage are reported via Datix and escalated appropriately.

- Coordinates medical workforce actions and supports clinical risk mitigation plans.

Service Group Nurse Director

- Provides senior nursing leadership and ensures safe nursing staffing and patient safety throughout the incident.

- Escalates nursing-related risks (e.g., environmental temperatures, call bell failures, infection control compromises).

- Supports Silver Command and ensures all nursing-related decisions and actions are logged.

- Oversees nursing actions during recovery, patient relocation, and reopening of affected areas.

Site/Bed Manager

- Takes immediate control until senior managers arrive.

- Confirms the extent and location of the outage and its impact on clinical services.

- Communicates with Estates/Kier, On-Call Managers, and Switchboard to coordinate alerting and escalation.

- Supports the setup of Command & Control and provides essential site-level situational intelligence.

- Coordinates real-time actions such as service moves, patient safety checks, and staffing changes.

Business Support/Loggist

- Manages all administrative tasks within Silver Command, including attendance, documentation, and action tracking.

- Loggist to complete formal log-keeping using national Loggist methodology.

Estates/Kier

- Leads technical diagnosis, isolation, and restoration of utilities.

- Manages generators, fuel supply, temporary power, and safety of electrical systems.

- Liaises directly with external utilities providers (electricity operator, Welsh Water, Gas Networks).

- Provides restoration timescales and risk assessments to Silver.

PCTSG

Will support with the planning, preparedness, response & recovery to any emergency by:

- Contributing to Planning & Preparedness

- Co-ordination & Communication of EPRR within your respective services

- Resilience, Escalation,  Response & Recovery Processes, (particularly in terms of business continuity management within your service)

- Collaboration with External Agencies for your respective service workstreams

- Monitoring & Evaluation through HB & service specific debriefs

MH&LD SDG

Will support with the planning, preparedness, response & recovery to any emergency by:

- Contributing to Planning & Preparedness

- Co-ordination & Communication of EPRR within your respective services

- Resilience, Escalation,  Response & Recovery Processes, (particularly in terms of business continuity management within your service)

- Collaboration with External Agencies for your respective service workstreams

- Monitoring & Evaluation through HB & service specific debriefs

### Clinical Leads

Maintain clinical safety and continuity of patient care by undertaking the following:

- Making patient safety decisions

- Determine service prioritisation or suspension

- Coordination of all clinical communications

### Clinical Leads (Bronze) - Roles and Responsibilities

### Patient Safety Decisions

- Assess and manage immediate patient safety risks within the clinical areas arising from the loss of utilities, prioritising high-risk and dependent patients.

- Escalate time-critical risks and safety thresholds to SDG Silver Command with clear clinical advice.

### Service Prioritisation or Suspension

- Maintain oversight of local service activity, capacity, and pressures during the incident.

- Identify and report areas where services are no longer safe to continue at current levels.

- Provide real-time clinical intelligence and recommendations to Silver Command to inform organisational decisions.

### Clinical Communications

- Ensure frontline teams receive and understand agreed clinical instructions and changes to practice.

- Ensure relevant clinical information and emerging issues at ward and service level are communicated promptly to Silver Command.

- Promptly report emerging risks, deterioration, or incidents through the command structure.

### Digital

### The same core principles apply as noted above for each SDG, in addition:

- Work to maintain and/or restore affected systems, networks, and clinical applications.

### Support Services

### The same core principles apply as noted above for each SDG

### Paediatrics

### The same core principles apply as noted above for each SDG, in addition:

- Confirm continuity of specialist equipment (incubators, paediatric ventilators).

### IP&C

### The same core principles apply as noted above for each SDG, in addition:

- Assess risks posed by utility loss to hygiene, isolation, air handling, and clinical environment integrity.

- Advise on safe operation of theatres, wards, and procedures.

### Procurement

### The same core principles apply as noted above for each SDG, in addition:

- Expedite emergency purchasing for generators, bottled water, medical gases, UPS battery units, or specialist engineering supplies.

- Liaise with suppliers for urgent delivery or alternative supply chains.

- Support financial approvals and invoicing during the incident.

### Medical Physics & Clinical Engineering

### The same core principles apply as noted above for each SDG, in addition:

- Ensure safety and continuity of clinical equipment, particularly life‑critical devices.

- Support rapid redeployment, testing, and repair of affected medical devices.

### Pathology (Laboratory Medicine, Cellular Pathology, Microbiology, Mortuary)

### The same core principles apply as noted above for each SDG, in addition:

- Assess impact of utilities on analysers, refrigeration, blood storage, and sample processing.

- Prioritise urgent testing and communicate any service constraints.

- Assess the impact on mortuary capacity and refrigeration.

### Radiology

### The same core principles apply as noted above for each SDG, in addition:

- Assess imaging equipment safety, shutdown procedures, and restart capability.

- Prioritise urgent/emergency imaging pathways.

- Escalate risks related to cooling loss, power stabilisation, or digital imaging system failures.

### Volunteer Services

### The same core principles apply as noted above for each SDG, in addition:

- Support non‑clinical duties such as patient support, wayfinding, welfare tasks, and logistical help (where safe).

### Care After Death Service

### The same core principles apply as noted above for each SDG, in addition:

- Support bereavement processes during operational disruption.

### ED, AMU & Old Persons Assessment Unit (OPAU)

### The same core principles apply as noted above for each SDG

### Directorate of Information, Communication & Engagement (DICE)

### The same core principles apply as noted above for each SDG, in addition:

- Coordinate all staff, partner and public messaging

- Ensure consistent lines of communication across digital, intranet, media and social platforms

### Medical Electronics (MEMS)

### The same core principles apply as noted above for each SDG, in addition:

- Perform rapid assessment of patient monitoring and bedside equipment.

- Provide device repair and replacement support.

### Renal Technical Services

### The same core principles apply as noted above for each SDG, in addition:

- Ensure continuity of dialysis plant, RO units, water purification systems.

- Monitor any risks that may threaten dialysis provision.

- Support patient transfer or regional escalation if needed.

### Nursing / Clinical Director representative, in addition:

### The same core principles apply as noted above for each SDG

- Provide senior clinical governance advice to Gold Command.

- Coordinate nursing impact assessments across sites/services.

- Support safe staffing decisions.

## 7. DETECTION & ALERTING

Effective early warning is critical for preserving patient safety, protecting essential infrastructure, and supporting coordinated command‑and‑control activation.             SBUHB will maintain mechanisms to ensure timely awareness of utilities or power disruption, including:

- Generator status monitoring via Building Management System (BMS), local control panels. Immediate escalation triggers for any generator start failure or significant performance anomalies.

- Local alert arrangements via local alarms and panels for electrical switchgear, water treatment plant alerts, medical gas plant and lift alarms monitored. Ward and department escalation routes ensuring frontline staff report unusual system behaviours at the earliest stage.

- Defined inspection and logbook checks via daily, weekly and monthly pre-planned maintenance (PPM) checks of critical plant including water quality checks, electrical switchgear visual inspections and air handling unit checks.

- Clear reporting routes into Site Management to ensure rapid situational awareness and coordinated responses. All alerts technical or operational must be reported immediately to the Site Manager so that they will assess and escalate based on the escalation frameworks; Estates on-call; Clinical leads; Silver/Gold on-call or EPRR as required.

## 8. IMMEDIATE RESPONSE

### The immediate response phase focuses on rapid situational awareness, protection of life‑critical services, and prompt escalation to ensure patient safety while technical investigations and restoration commence.

### First 0–15 minutes

### Objective: Establish control, protect life, and initiate escalation.

- Confirm extent and cause of outage

- Protect life-critical services

- Notify as per escalation routes

### Confirm Extent and Cause of Outage

- Estates to rapidly assess:

  - Whether the outage is localised or site-wide

  - Affected utilities (power, water, medical gases, HVAC, IT)

  - Likely cause (network failure, plant failure, external supply disruption, planned outage escalation)

- Verification through:

  - BMS alarms / local panels

  - Generator and plant control interfaces

  - Reports from wards/departments and Security/Site Management

- Initial findings communicated immediately to Site Management.

### Protect Life-Critical Services

- Ensure emergency power systems have engaged and are performing as expected.

- Confirm continuity of supply to:

  - Critical Care, ED, Theatres, Neonatal & Paediatrics

  - Dialysis, Medical Gases, Imaging, Laboratories

  - Fire systems, emergency lighting, communications

- Implement immediate load-shedding if required to preserve generator capacity.

- Clinical Leads to assess any immediate patient risk and initiate contingency actions.

### Notify as per Escalation Routes

- Immediate notification to:

  - Site Manager

  - Estates Senior On-Call

  - Silver On-Call / Gold On-Call (as appropriate)

- Initial incident logged within Silver/Gold Command or Datix (where applicable).

- EPRR notified if a BC/Critical incident is declared or a prolonged disruption.

### Where a loss, degradation or contamination of water supply is suspected or confirmed, managers and response teams should refer to the Bitesize Water and Sanitation Response Scripts (Appendix H) to support immediate actions, staff briefing, patient safety messaging and the implementation of hygiene and sanitation control measures.

### 15 minutes – 4 hours

### Objective: Stabilise services, confirm endurance, and prepare for escalation or recovery.

- Confirm generator load and fuel endurance

- Prioritise clinical services

- Prepare for escalation if restoration is delayed

Confirm Generator Load and Fuel Endurance

- Estates to:

  - Verify generator loading against rated capacity

  - Identify non-essential circuits for disconnection

  - Confirm fuel volumes, burn rate, and projected endurance

- Fuel endurance estimates to be shared with:

  - Respective SDG/Site Management

  - Silver Command

- Immediate actions taken if endurance is insufficient (e.g. fuel resupply requests).

Prioritise Clinical Services

- Clinical Leads and Nursing Directors to:

  - Prioritise emergency, urgent, and life-saving activity

  - Defer elective or non-urgent services where appropriate

- SDGs to activate Business Continuity Procedures for affected services.

- IP&C to advise on infection control and environmental safety risks.

Prepare for Escalation if Restoration Is Delayed

- Establish recovery timelines with utility providers or contractors.

- Silver Command to consider:

  - Stand-up of Tactical Command

  - Formal BC/Critical incident declaration (if thresholds met)

- Communications team to:

  - Prepare staff briefings and holding statements

  - Support internal staff awareness and reassurance

- Resource planning initiated for staffing, welfare, and equipment resilience.

## 9. SUSTAINED OUTAGE MANAGEMENT

For incidents extending beyond initial response windows, SBUHB will adopt a structured, endurance-based management approach, focused on resilience, staff welfare, and safe continuity of care.

For extended incidents:

- Fuel supply resilience and re-supply

- Temporary generator options

- Staff welfare and fatigue

- Consideration of patient decant or service suspension

- Ongoing Command, Review and Recovery

Fuel Supply Resilience and Re-Supply

- Estates to:

  - Secure priority fuel deliveries via framework suppliers

  - Confirm delivery routes, access arrangements, and pump compatibility

  - Maintain minimum fuel thresholds for critical sites

- Procurement to support emergency purchasing and contractual escalation.

- Silver Command to receive regular fuel endurance SITREPs.

Temporary Generator and Technical Mitigation Options

- Assess feasibility of:

  - Mobile or trailer-mounted generators

  - Temporary power feeds or bypass arrangements

- Ensure:

  - Compliance with electrical safety and HTM requirements

  - Controlled connection via Estates with Permit to Work

- Prioritise deployment to:

  - Critical clinical areas

  - Medical gas plant

  - Digital and communications systems

Staff Welfare and Fatigue Management

- Operational leads to:

  - Monitor staff fatigue arising from prolonged disruption

  - Implement rest rotations, breaks, and welfare provisions

- Support Services to:

  - Provide catering, hydration, and rest areas

  - Support transport arrangements if required

- Workforce & OD to support redeployment and wellbeing measures.

Consideration of Patient Decant or Service Suspension

- Clinical and Executive Leads to assess:

  - Ongoing risk to patient safety

  - Environmental integrity (power, water, ventilation)

  - Ability to sustain care safely

- Options may include:

  - Internal decant of patients to unaffected areas

  - Inter-site transfers within Health Board or region

  - Temporary service suspension or diversion

- All decisions to be:

  - Clinically led

  - Clearly documented

  - Communicated to staff, patients, partners, and Welsh Government

- EPRR to coordinate regional and national liaison where required.

Ongoing Command, Review and Recovery

- Maintain Silver / Gold Command as required.

- Regular review of:

  - Utility restoration progress

  - Clinical risks

  - Operational sustainability

- Formal de-escalation only once services are stable and risks mitigated.

- Recovery phase to include:

  - Equipment checks

  - Deferred activity planning

  - Post-incident debrief

The Bitesize Water and Sanitation Response Scripts (Appendix H) should be used to inform consistent internal messaging to staff and clinical teams during water‑related incidents.

Public communications arising from utilities failures will be coordinated in line with Health Board Crisis Communications arrangements. Where appropriate, escalation to PAWS mechanisms, as set out in (Appendix I), will be considered to support timely, clear and consistent public safety messaging

## 10. VULNERABLE POPULATION, CRITICAL SERVICES & CLINICAL       PRIORITISATION

Clinical services will be categorised to inform decision-making during the response phase (see above Clinical Leads roles & responsibilities):

- Life-sustaining services

- Critical but deferrable services

- Elective / non-urgent services

In addition, clinical decisions will remain clinically led, supported by command structures in relation to the below:

- Identification of vulnerable patients, home oxygen users, dialysis patients etc.

- Mapping of critical infrastructure requiring uninterrupted power/water.

- Align with guidance that emphasises heightened risk for elderly and clinically vulnerable populations during outages.

In incidents involving water supply interruption or contamination, clinical decision‑making and infection prevention measures will be supported by the Bitesize Water and Sanitation Response Scripts (Appendix H), which provide practical prompts to maintain patient safety, hygiene standards and continuity of care, particularly for vulnerable and high‑risk patient groups.

## 11. CONTRACTOR CONTROL & SAFETY

Swansea Bay University Health Board (SBUHB) will ensure that all contractor activities undertaken during utility disruptions, emergency works, or recovery phases are strictly controlled, safely managed, and fully compliant with statutory requirements, Welsh Health Technical Memoranda (WHTMs), and local electrical safety procedures.
Contractor control is critical during emergency conditions, where increased risk exists due to live systems, temporary connections, abnormal operating states, and time-critical pressures. All electrical works must follow:

- Formal Permit to Work (PTW) processes.

PTWs must clearly define:

  - Scope of work

  - Isolation points and earthing arrangements

  - Status of supplies (normal, essential, generator-fed)

  - Identified hazards and control measures

  - Start and completion times

  - Authorised Person oversight - All contractor electrical works must be carried out under the direct control and supervision of an appropriately appointed Authorised Person (AP) in accordance with HTM electrical safety management arrangements.

SBUHB maintains a clear and absolute authority to stop works where safety, isolation integrity, or service continuity is compromised.

Emergency conditions do not override PTW requirements; however, expedited processes may be used where formally approved by Estates leadership.

Contractor Competency and Induction

All contractors must:

- Demonstrate appropriate competence, qualifications, and authorisation for the task

- Provide method statements and risk assessments relevant to the emergency works

Contractors must receive:

- Site-specific induction (including emergency arrangements, escape routes, and escalation routes)

- Briefing on the current operational status (e.g. generator operation, partial supplies, load shedding)

Contractors unfamiliar with healthcare environments must be closely supervised at all times.

Control of Emergency and Temporary Works

Where emergency or temporary solutions are required:

- Temporary power arrangements must:

  - Be formally designed or agreed by Estates

  - Meet electrical safety and HTM standards

  - Be clearly labelled and documented

- No temporary arrangement may:

  - Bypass safety systems

  - Introduce unacceptable risk to patients, staff, or infrastructure

- Temporary installations must be reviewed regularly and removed once permanent restoration is achieved.

## 12. RECOVERY & STAND-DOWN

The recovery phase of an incident needs to be considered at the onset of an emergency. The recovery phase continues until disruption has been rectified, demands on services have returned to normal and the needs of the affected population have been met.

Recovery management should encompass the physical, social, psychological, political and financial consequences of an emergency.

It is the responsibility of the Strategic/Tactical Commander to anticipate consequences and appropriate recovery planning right from the beginning of any response and a recovery team should be convened to manage the recovery of an incident.

Consideration will also be required for sustainability in the event of a protracted incident.  People who are acting in roles designated in emergency response plans should consider handing over the role to a colleague of similar seniority and experience either at the time they have to leave the Health Board due to other commitments, or at the time when they feel that their effectiveness may become impaired. During these circumstances a rolling rota will be required.

The Chief Operating Officer (COO) will refer to their respective recovery action card and will also consider the below:

- Utility system restoration timelines

- Recommissioning clinical equipment

- Water system flushing & validation

- Gas safe reconnection procedures (aligned with IGEM/GL/9 and HSE principles).

- Confirmation of stable utility supplies

- Generator inspection and reset (including any other utility inspections that may be required)

- Clinical restoration

- Stand-down advised by Silver & agreed by Gold

Confirmation of Stable Mains Supply (Estates Lead)

Before any stand-down from emergency arrangements:

- Estates will formally confirm that:

  - Normal mains utility supply has been restored, is stable, and operating within design parameters.

  - There is no ongoing reliance on emergency or temporary systems, unless explicitly agreed and risk-assessed.

  - Utility providers have confirmed restoration is secure and not subject to further planned interruption.

- Verification will include:

  - Physical inspection of incoming supplies and plant

  - Review of BMS trends and alarms

  - Confirmation that no abnormal switching arrangements remain in place

- No transition out of incident response will occur until stability is verified, not merely supply reinstated.

Generator Inspection, Load Removal and Reset (Estates Lead)

Following restoration of normal supply:

- Estates will:

  - Safely transfer loads back to mains power in a controlled and sequenced manner.

  - Inspect all generators that have operated during the incident for:

    - Mechanical integrity

    - Alarms, faults, or abnormal performance

    - Fuel levels and contamination risks

  - Conduct post-run checks in line with manufacturer guidance and HTM requirements.

- Generators will not be returned to “available” status until:

  - All faults are cleared or managed

  - Fuel tanks are replenished to resilience levels

  - Any temporary connections are removed

- Any defects or performance issues will be:

  - Logged

  - Risk-assessed

  - Scheduled for remedial action as part of recovery works.

Clinical Restoration and Service Normalisation (COO & Clinical Leads)

- The COO, supported by Clinical Directors, Nursing leadership, and Service Delivery Groups, will:

  - Confirm when it is clinically safe to return services to normal operation.

  - Authorise phased restoration of:

    - Elective activity

    - Diagnostics and investigations

    - Outpatient and procedural services

- Clinical areas must confirm:

  - Equipment functionality following power restoration

  - Environmental safety (temperature, ventilation, water, medical gases)

  - Digital and diagnostic system integrity

- Any service that was suspended, decanted, or diverted will not restart automatically; restoration must be explicitly agreed and documented.

## 13. LEARNING & ASSURANCE

## These processes will be aligned to the following SBUHB EPRR, Estates and Digital documents:

- HB & EPRR Lessons Identified (LID) Process Policy

- EPRR & Recovery Operating Framework

- EPRR Training & Exercise Operating Framework

- Estates Black Start Exercises

- Estates/Digital Maintenance Programmes

14. TRAINING & EXERCISING

## SBUHB recognises that effective response to utility failure and power disruption depends on regular training, exercising, and organisational learning. This framework will therefore be supported by a structured programme of testing, multi-disciplinary exercises, and formal review to ensure staff remain competent, confident, and prepared This will include:

### Annual Black Start Testing (Estates Lead)

### SBUHB will undertake formal annual black start testing at all acute hospital sites, led by Estates and overseen through established compliance and governance arrangements.

- Tests will:

  - Simulate total loss of mains supply

  - Confirm generator auto-start, load acceptance, and system resilience

  - Test switching arrangements, essential systems prioritisation, and recovery processes

- Testing will be:

  - Pre-planned and risk-assessed

  - Conducted in line with HTM requirements and manufacturer guidance

  - Communicated in advance to Site Management and clinical leadership

- Outcomes will be:

  - Documented in Estates records and compliance systems

  - Reviewed for lessons learned and remedial actions

  - Used to inform capital, maintenance, and resilience investment planning

- Significant failures or risks identified will be escalated immediately through Estates governance and executive routes.

Multi-disciplinary loss of utilities exercises

### Exercises will:

- Involve Estates, Site Management, Clinical Leads, COO, EPRR, Digital, Communications, and Support Services

- Test:

  - Detection and alerting processes

  - Escalation routes and command structures (Bronze/Silver/Gold)

  - Clinical prioritisation and patient safety decisions

  - Communications and partner liaison

- Be delivered through a mix of:

  - Tabletop exercises

  - Command post simulations

  - Live or semi-live technical scenarios (where safe)

- Include participation from external partners where appropriate (e.g. Welsh Government, LRF representatives, utility providers).

### Training for Key Roles

### Targeted training will be provided for staff with defined responsibilities under this framework, including:

- Estates Authorised Persons and engineers

- Site Managers

- On-call executives and senior managers

- Clinical Leads and Nursing leadership

- EPRR and Communications staff

### Training for Key Roles

### Targeted training will be provided for staff with defined responsibilities under this framework, including:

- Estates Authorised Persons and engineers

- Site Managers

- On-call executives and senior managers

### Training will focus on:

- Roles and responsibilities during utility failure

- Use of escalation frameworks and command structures

- Decision-making under emergency and endurance conditions

- Patient safety and risk-based prioritisation

# 15.DOCUMENT REFERENCES

This document should be read in conjunction with the following documents:

- SBUHB Major Incident Procedures

- SBUHB Business Continuity Procedures

- SBUHB Infection Prevention and Control Policies

- SBUHB Contaminated Casualties Procedure

- SBUHB Water Safety Plans

- UK Government Guidance: Preparing for and Responding to Energy Emergencies (Appendix F)

- National Fuel Plan (Appendix G)

- Public Health Advice – Water & Sanitation during a national power outage (Appendix H)

- UK Government Guidance: Written Science Annex - National Power Outage (Appendix I)

- HTM / WHTM 06‑01 Electrical Services Supply and Distribution (Appendix J)

- National Emergency Plan – Downstream Gas and Electricity (Appendix K)

- South Wales Police MACG (Appendix L)

### 16. Glossary of Terms

AMU – Acute Medical Admissions Unit

AP – Authorised Person

BC – Business Continuity

BCM – Business Continuity Management

BCP – Business Continuity Procedure

BMS - Building Management System

C3 - Command, Control & Co-ordination

CHDU – Cardiac High Dependency Unit

CITU – Cardiac Intensive Therapies Unit

COO - Chief Operating Officer

DICE - Directorate of Information, Communication & Engagement

DNO - Distribution Network Operator

DWI - Drinking Water Inspectorate

ECU - Enhanced Care Unit

ED – Emergency Department

EPRR - Emergency Preparedness, Resilience & Response

HB – Health Board

H/ICC – Hospital/Incident Co-ordination Centre

HSE – Health & Safety Executive

HTM - Health Technical Memorandum

HVAC - Heating, ventilation and Air Conditioning

IGEM - International Genetically Engineered Machine

IP&C – Infection Prevention & Control

IT – Information Technology

ITU - Intensive Therapies Unit

JDM - Joint Decision-Making Model

JESIP – Interoperability Principles

LID – Lessons Identified

MACG - Multi-Agency Co-ordination Group

MEMS – Medical Electronics

MH&LD - Mental Health & Learning Disabilities

MIU – Minor Injuries Unit

NHS – National Health Organisation

NPO - National Power Outage

NPTSSG – Neath Port Talbot & Singleton Service Delivery Group

OPAU - Old Persons Assessment Unit

PAWS - Pre-Agreed Written Science

PCTSG – Primary Care & Therapies Service Delivery Group

PPM - Pre-planned maintenance

PTW – Permit to Work

RCG - Recovery Coordinating Group

RO units - Reverse Osmosis units

SALTO - Electronic locks that enables users to unlock doors with a range of secure credentials and mobile technologies

SAMM - Situational Awareness Multi-Agency Meeting

SBUHB - Swansea Bay University Health Board

SCG - Strategic Coordinating Group

SDG – Service Delivery Group

SWLRF - South Wales Local Resilience Forum

SWP - South Wales Police

TCG - Tactical Coordinating Group

UK – United Kingdom

UPS - Uninterruptible Power System

VoIP - Voice over Internet Protocol

WOD - Workforce & Organisational Development

### 17. APPENDICES

APPENDIX A – Loss of Utilities Decision Log

APPENDIX B - Generator Status Checklist

APPENDIX C - Contractor Permit to Work template

APPENDIX D - Crisis Communications Plan

APPENDIX E - Fuel monitoring log

APPENDIX F – Preparing for and Responding to Energy Emergencies

Preparing for and responding to energy emergencies - GOV.UK

APPENDIX G – National Fuel Plan

National Emergency Plan for Fuel: summary of response tools - GOV.UK

APPENDIX H – Public Health Advice – Water & Sanitation during a national power outage

APPENDIX I – UK Government Guidance: Written Science Annex - National Power Outage

APPENDIX J - Health Technical Memorandum 06-01: Electrical services supply and distribution

Health Technical Memorandum 06-01: Electrical services supply and distribution (2017 edition)

APPENDIX K – National Emergency Plan – Downstream Gas & Electricity

National Emergency Plan 2023: Downstream gas and electricity

APPENDIX L – South Wales Police MACG

APPENDIX M – JESIP Interoperability Principles & Joint Decision-Making Model